SLIM Full-Mesh Benchmark

Test Report

Repository slim-benchmarks  ·  branch feat/full-mesh-scale-bench

# 1. Purpose and scope

This report records the testing of the four-cell full-mesh transport benchmark: what was tested, how each test was validated, the measured results, and the issues found and fixed during testing. Every cell is a full mesh where each agent exchanges a message with every other agent. Two protocols (A2A, HTTP) are each tested with and without SLIM, giving four cells.

# 2. Test environment

- Machine: Apple MacBook Pro (Mac16,8), 12 CPU cores, 24 GB RAM, macOS.

- HTTP and SLIM mesh cells (3, 4): Python 3.12 (.venv-mesh), slim-bindings 1.3.0, node ghcr.io/agntcy/slim:1.3.0.

- A2A cells (1, 2): Python 3.13 (.venv-a2a), agntcy-app-sdk 0.5.1, a2a-sdk 0.3.20, slim-bindings 1.1.1, node ghcr.io/agntcy/slim:1.0.0 with the Aether server config.

- All runs are loopback on one machine, so latencies are a lower bound; comparisons between cells remain valid.

# 3. Test matrix

| Cell | Transport | Node | Evidence file |
| --- | --- | --- | --- |
| 1 | A2A, no SLIM | none | a2a_http_mesh_sweep.json |
| 2 | A2A over SLIM | slim:1.0.0 | a2a_slim_mesh_sweep.json |
| 3 | HTTP, no SLIM | none | http_mesh_sweep_parallel.json |
| 4 | HTTP over SLIM | slim:1.3.0 | http_slim_mesh_sweep.json |

# 4. Test method

Each agent is a real participant. For HTTP it is an HTTP server with a kept-alive connection to every peer; for the SLIM mesh it is a slim-bindings app holding a point-to-point session to every peer; for A2A it is an echo A2A agent (server plus client) running in its own process. Sessions and connections are opened once in a warm phase, so the timer measures the send path, not setup.

Each cell is exercised in up to three execution modes:

- Sequential: one message in flight at a time (latency floor).

- Concurrent: every agent fans out to all peers at once using threads.

- Parallel: one operating-system process per agent (true parallelism); applied to cells 3 and 4 and the SLIM mesh primitive. A2A cells already run process per agent.

Message count for N agents is N x (N-1) per round. Pass criterion for every run: receivers count messages and the total reconciles exactly against messages sent, with zero loss.

# 5. Delivery verification

Every configuration across all five sweeps reconciled sent against delivered with no loss. Representative counts per agent size:

| Config (per run) | Sent | Delivered | Loss |
| --- | --- | --- | --- |
| 5 agents x rounds | 60 | 60 | 0 |
| 10 agents x rounds | 270 | 270 | 0 |
| 20 agents x rounds | 1140 | 1140 | 0 |

Result: 100 percent delivery in every configuration and every mode tested.

# 6. Latency results

Per-message latency, mean milliseconds, 64-byte payload. Lower is better.

## 6.1 SLIM mesh primitive (point-to-point unicast)

| Agents | sequential | concurrent | parallel |
| --- | --- | --- | --- |
| 5 | 0.070 | 0.349 | 0.174 |
| 10 | 0.064 | 0.753 | 0.104 |
| 20 | 0.073 | 1.819 | 0.699 |

## 6.2 Cell 3 — HTTP, no SLIM

| Agents | sequential | concurrent | parallel |
| --- | --- | --- | --- |
| 5 | 0.431 | 1.757 | 1.526 |
| 10 | 0.220 | 2.799 | 1.820 |
| 20 | 0.229 | 3.011 | 2.160 |

## 6.3 Cell 4 — HTTP over SLIM

| Agents | sequential | concurrent | parallel |
| --- | --- | --- | --- |
| 5 | 0.159 | 0.913 | 1.080 |
| 10 | 0.137 | 1.603 | 1.272 |
| 20 | 0.132 | 3.892 | 2.210 |

## 6.4 Cell 1 — A2A, no SLIM (round trip)

| Agents | sequential | concurrent |
| --- | --- | --- |
| 5 | 3.074 | 5.967 |
| 10 | 4.073 | 18.927 |
| 20 | 7.448 | 58.850 |

## 6.5 Cell 2 — A2A over SLIM (round trip)

| Agents | sequential | concurrent |
| --- | --- | --- |
| 5 | 9.693 | 19.669 |
| 10 | 11.939 | 45.821 |
| 20 | 18.549 | 148.341 |

# 7. Cross-cell comparison

## 7.1 HTTP with vs without SLIM (sequential round trip)

| Agents | HTTP no SLIM | HTTP over SLIM | Faster by |
| --- | --- | --- | --- |
| 5 | 0.431 | 0.159 | 63 % |
| 10 | 0.220 | 0.137 | 38 % |
| 20 | 0.229 | 0.132 | 42 % |

HTTP over SLIM is faster than direct HTTP, about 42 percent at 20 agents, consistent with the previously reported figure near 33 percent.

## 7.2 A2A with vs without SLIM (sequential round trip)

| Agents | A2A no SLIM | A2A over SLIM | Slower by |
| --- | --- | --- | --- |
| 5 | 3.074 | 9.693 | 3.2x |
| 10 | 4.073 | 11.939 | 2.9x |
| 20 | 7.448 | 18.549 | 2.5x |

On this single machine A2A over SLIM is slower than A2A without SLIM, because the SLIM RPC and per-call session handshake add fixed overhead that the cheap loopback HTTP path does not pay. The SLIM benefit for A2A is expected at larger scale or over a real network, which is follow-up work.

# 8. Resource usage and bottleneck

Resource usage was sampled with docker stats (node container) and ps and top (host and client processes) during representative loads on the 12-core, 24 GB machine.

| Component | Memory | CPU | Bottleneck? |
| --- | --- | --- | --- |
| SLIM node (broker) | 6-17 MB | 0-68% of 1 core | No |
| Client process, HTTP/SLIM | ~44 MB each | shares host cores | Yes (CPU) |
| Client process, A2A | ~136 MB each | shares host cores | Yes (CPU+mem) |
| Loopback network | KB-scale | negligible | No |

Across modes, client CPU usage tracks how the work spreads over cores:

| Mode | Cores used | Note |
| --- | --- | --- |
| Sequential | ~1.5 | 1 Python thread plus SLIM Rust runtime; GIL-bound |
| Concurrent (threads) | ~1.5 effective | GIL serializes Python; OS thread limit near 80 agents |
| Parallel (processes) | up to all 12 | true parallelism; host hits 0% idle; OS time-slices beyond core count |

Findings:

- The SLIM node is not the bottleneck. It stays light, tens of megabytes of memory and at most about one core, even under the heaviest concurrent fan-out.

- The network is not the bottleneck. Traffic is loopback and totals only kilobytes; it is never the limiting factor in these runs.

- Client-side CPU is the primary bottleneck. In sequential and thread-concurrent modes the Python interpreter lock caps useful work at roughly 1.5 cores regardless of agent count; only the process-per-agent parallel mode uses all cores, at which point the host reaches 0 percent idle.

- At higher agent counts the limit shifts to host memory and process or thread count. Each HTTP/SLIM agent process is about 44 MB and each A2A agent process is about 136 MB, so A2A becomes memory-bound near 50 agents on 24 GB. The thread-concurrent mode separately hits the operating-system thread limit and fails to start new threads at around 80 agents, which is why scaling past that point uses the sequential or parallel modes.

# 9. Issues found and fixed during testing

- Process-parallel SLIM mesh failed with 'no matching route' until the sender declared a route to each peer (set_route) before opening the session; the single-connection thread model did not need this.

- Thread-based concurrent mode showed inflated p99 tails (HTTP up to 63 ms at 20 agents) due to the Python interpreter lock; the process-parallel mode removed this (p99 about 3.5 ms), confirming the tail was a lock artifact, not transport.

- A2A stack failed to start ('Expected UnaryUnaryHandler subclass') under loosely resolved dependencies; pinning to the lockfile versions (agntcy-app-sdk 0.5.1, slim-bindings 1.1.1, a2a-sdk 0.3.20) fixed it.

- A2A client and server in the same process failed the SLIM handshake because the client reused the server connection; running them as separate processes (the mesh topology) resolved it.

# 10. How to reproduce

- Start the SLIM node for the cell under test (1.3.0 for cells 3/4, 1.0.0 with the Aether config for cell 2).

- Run the cell's sweep, for example HTTP over SLIM:

PYTHONPATH=. .venv-mesh/bin/python http_slim_mesh_benchmark.py --sweep \

--agents-list 5,10,20 --payloads 64,512 --rounds 3 \

--output docs/evidence/http_slim_mesh_sweep.json

- Each runner prints sent vs delivered per row and writes the evidence JSON used in this report.

# 11. Limitations

- Loopback on one machine: latencies are a lower bound and the network is not exercised.

- True simultaneity caps at the core count (12); larger agent counts are partly time-sliced.

- SLIM measures a one-way publish to confirmed delivery while HTTP and A2A measure a full round trip; aligning the measurement basis is open.

- A2A cells use an echo agent, so they measure transport only, not semantic-negotiation convergence.